Computer Organization and Architecture (EET2211)

LAB I: Examine & Analyze Different Addressing Modes of 8086 Microprocessor

Siksha ‘O’ Anusandhan Deemed to be University, Bhubaneswar

| Branch: Section: |  |  |  |
| --- | --- | --- | --- |
| S. No. | Name | Registration No. | Signature |
| 30. | Aishwary singh gaharwar | 1941017093 |  |

Marks: ______/10

Remarks:

Teacher’s Signature

I. OBJECTIVE:

- Addition of two 16bit numbers using immediate addressing mode.

- Addition of two 16bit numbers using direct addressing mode.

- Addition of two 16bit numbers using indirect addressing mode.

- Addition of two 16bit numbers using index addressing mode.

- Addition of two 16bit numbers using base index addressing mode.

II. PRE-LAB

For Obj. 1:

a. Explain immediate addressing mode briefly -  The mode in which the data operand is a part of the instruction itself is known as immediate addressing mode. Ex- MOV AX, 0005h.

b. Examine & analyze the output obtained from addition of two 16 bit numbers- The Addressing mode In which the data operand is a part of the instruction itself.

EX-MOV AX,0005H(16 bit)

c. Write the assembly code -

MOV AX,5966H

MOV BX,4322H

ADD AX, BX

HLT

For Obj. 2:

- Explain direct addressing mode briefly - The mode in which the effective address of the memory location is written directly in the instruction. Ex- MOV AX, [5000H].

- Address {(10*DS)H+5000H)}

b. Examine & analyze the output obtained from addition of two 16 bit numbers-

c. Write the assembly code-

MOV AX,[3000H]

MOV BX,[2000H]

ADD AX,BX

HLT

For Obj. 3:

a. Explain indirect addressing mode briefly- The addressing mode allows data to be addressed at any memory location through an offset address held in any of following: BP,BX,DI and SI.

EX-MOV AX,[BX]

b. Examine & analyze the output obtained from addition of two 16 bit numbers-

c. Write the assembly code-

MOV AX,0000H

MOV DS,AX

MOV BX,0500H

MOV AX,[BX]

INC BX

INC BX

MOV CX,[BX]

ADD AX,CX

INC BX

INC BX

MOV [BX],AX

HLT

For Obj. 4:

a. Explain index addressing mode briefly-In this addressing mode the operand offset is found by adding the contents of SI or DI register and 8-bit/16-bit displacements.

Ex- MOV AX, [SI]

b. Examine & analyze the output obtained from addition of two 16 bit numbers-

c. Write the assembly code-

MOV AX,0000H

MOV DS,AX

MOV SI,0500H

MOV AX,[SI+0]

MOV BX,[SI+2]

ADD AX,BX

MOV [SI+4],AX

HLT

For Obj. 5:

a. Explain base index addressing mode briefly- In this addressing mode the operands offset is computed by adding the base register contents. An index register contains an 8bit or 16bit displacement

Ex-MOV AX,50H[BX][SI]

b. Examine & analyze the output obtained from addition of two 16 bit numbers-

c. Write the assembly code-

MOV AX,0000H

MOV DS, AX

MOV BX,1234H

MOV SI,0500H

MOV CX, [BX+SI]

MOV DX, [BX+SI+02]

ADD AX, CX

ADD AX, DX

HLT

IV. POST LAB:

            - Discuss different general-purpose registers used in 8086 microprocessor.

Ans- EU has 8 general purpose registers. Can be individually used for storing 8- bit data. AL register is also called Accumulator. Two registers can also be combined to form 16-bit registers. The valid register pairs are – AX, BX, CX, DX.

AX -) Word multiply, word divide, word I/O

AL -) Byte multiply, byte divide, byte I/O, decimal arithmetic

AH -) Byte multiply, byte divide BX Store address information

CX -) String operation, loops CL Variable shift and rotate

DX -) Word multiply, word divide, indirect I/O (used to hold I/O address during I/O instructions. If the result is more than 16-bits, the lower order 16-bits are stored in accumulator and higher order 6-bitsare stored in DX register)

            - Explain the concept of segmented memory. What are its advantages?

Ans- 8086 has a 20-bit address bus, So it can address a maximum of 1MB of memory. 8086 can work with only four 64KB segments at a time within this 1MB range.

These four memory segments are called:

i) CODE segment

(ii) STACK segment

(iii) DATA segment

(iv) EXTRA segment

CODE SEGMENT- The part of memory from where BIU is currently fetching instruction code bytes. It is used for storing the instructions.

STACK SEGMENT- A section of memory set aside to store address and data while a subprogram executes. It is sued as a stack and is used to store the return address.

DATA AND EXTRA SEGMENTS -Used for storing data values or data bytes to be used in the program

The size of each segment is 64 KB. A segment may be located any-where in the memory. Each of these segments can be used for a specific function.

Address of a segment is of 20-bits. A segment register stores only upper 16 bits of the starting address of the corresponding segments. The 1-bit contents of the segment registers in the BIU actually points to the starting location of a particular segment always inserts zeros for the lowest 4-bits of the 20-bit starting address. A 64-KB segment can be located anywhere in the memory, bus will start at an address with zero in the lowest 4-bits.

            - Explain the physical address formation in 8086.

Ans-) Offset value + Segment Register = Physical Address

Suppose, cs-34BA

IP-8AB4

Physical Address-34BA0 +

8AB4

3D654

            - Write a program to add two 16 bit numbers 12H and 08H, and store the sum.

MOV AX,12H

MOV BX,08H

ADD AX,BX

HLT

IV. POST LAB: